# 🔢 Arduino DIY Calculator using 4x4 Keypad & 16x2 LCD 📟

### 🚀 Overview

This is a simple yet functional calculator built using an Arduino Uno, a 4x4 Keypad, and a 16x2 LCD Display. It can perform basic arithmetic operations (+, -, *, /), supports decimal numbers, and includes error handling for invalid inputs (like division by zero).

## 🛠 Features

✅ Basic Arithmetic Operations → Addition, Subtraction, Multiplication, Division
✅ Decimal Support → Perform calculations with decimal numbers
✅ Error Handling → Prevents multiple operators, division by zero errors
✅ Clear Functionality → Reset calculations with a single press

## 📜 Components Required

| Component | Quantity |
| --- | --- |
| Arduino Uno | 1 |
| 16x2 LCD Display (with or without I2C) | 1 |
| 4x4 Keypad | 1 |
| 10KΩ Potentiometer (for LCD contrast) | 1 |
| Jumper Wires | Multiple |
| Breadboard | 1 |

## 🔌 Wiring Diagram

LCD Pins  →  Arduino Pins

VSS      →  GND

VDD      →  5V

V0       →  Middle Pin of Potentiometer

RS       →  13

RW       →  GND

E        →  12

D4       →  11

D5       →  10

D6       →  9

D7       →  8

A (LED)  →  5V

K (LED)  →  GND

Keypad Pins  →  Arduino Pins

Row 1 (R1)  →  7

Row 2 (R2)  →  6

Row 3 (R3)  →  5

Row 4 (R4)  →  4

Col 1 (C1)  →  3

Col 2 (C2)  →  2

Col 3 (C3)  →  1

Col 4 (C4)  →  0

## 💻 Arduino Code

#include <LiquidCrystal.h>

#include <Keypad.h>

LiquidCrystal lcd(13, 12, 11, 10, 9, 8);

float first = 0, second = 0, total = 0;

char customKey;

bool isSecondNum = false;

char operation = '\0';

const byte ROWS = 4;

const byte COLS = 4;

char keys[ROWS][COLS] = {

{'1','2','3','/'},

{'4','5','6','*'},

{'7','8','9','-'},

{'C','0','=','+'}

};

byte rowPins[ROWS] = {7 ,6 ,5 ,4};

byte colPins[COLS] = {3, 2, 1, 0};

Keypad customKeypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

void setup() {

lcd.begin(16, 2);

lcd.print("Arduino Calculator");

delay(2000);

lcd.clear();

}

void loop() {

customKey = customKeypad.getKey();

if (customKey) {

if ((customKey >= '0' && customKey <= '9') || customKey == '.') {

if (isSecondNum) {

second = (second * 10) + (customKey - '0');

} else {

first = (first * 10) + (customKey - '0');

}

lcd.print(customKey);

}

else if (customKey == '+' || customKey == '-' || customKey == '*' || customKey == '/') {

if (operation == '\0' && first != 0) {

operation = customKey;

isSecondNum = true;

lcd.print(customKey);

}

}

else if (customKey == '=') {

if (operation != '\0' && isSecondNum) {

switch (operation) {

case '+': total = first + second; break;

case '-': total = first - second; break;

case '*': total = first * second; break;

case '/':

if (second == 0) {

lcd.clear();

lcd.print("Err: Div by 0");

delay(2000);

resetCalculator();

return;

} else {

total = first / second;

}

break;

}

lcd.setCursor(0, 1);

lcd.print("= ");

lcd.print(total);

delay(3000);

resetCalculator();

}

}

else if (customKey == 'C') {

resetCalculator();

}

}

}

void resetCalculator() {

first = second = total = 0;

isSecondNum = false;

operation = '\0';

lcd.clear();

}

## 📷 Project Preview

## 📌 How to Use?

1️⃣ Power up the Arduino.
2️⃣ The LCD will display "Arduino Calculator".
3️⃣ Press numeric keys to input numbers.
4️⃣ Press +, -, *, or / to choose an operation.
5️⃣ Enter the second number and press = to see the result.
6️⃣ Press C to clear and start a new calculation.

## 📚 Libraries Used

📌 Keypad.h - Used for handling the 4x4 keypad.
📌 LiquidCrystal.h - Used to control the 16x2 LCD Display.

## 📜 License

📝 This project is open-source under the MIT License. Feel free to modify and improve!